Title: Study of a metasurface-based multi-functional polarization converter

Abstract: In the paper, we proposed a multi-functional microwave device to realize the polarization conversion of transport wave based on the anisotropic metasurface which was composed of a series of flower-liked unit cells separated by air spaces. The simulation results showed revealed that the metasurface possesses the abilities of quarter-wave plate around 7.8 GHz and half-wave plate within the frequency range from 8.8 to 11.1 GHz. The converter showed better conversion efficiencies for varying incidence angle at 7.8 GHz in comparison with the ones within 9-11 GHz. Meanwhile, the influences of structural parameters were also studied in detail. The transmissive converter would broaden the function of traditional antenna conveniently.

- Introduction

Polarization state is a fundamental characteristic of electromagnetic (EM) waves. With the development of satellite communication and antenna technology, different polarization states of EM waves are required in an integrated system, suggesting the need of polarization conversion. Traditional techniques mainly rely on the feature of the material, resulting in a narrow operation bandwidth. Meanwhile, to reach the desired phased difference of the two orthogonal components, a large volume is also required. To overcome these drawbacks and meet the requirement of integration, metasurface is introduced, which has been successfully demonstrated can tailor the fundamental characteristics of EM waves.

The metasurface is commonly composed of single or multi-layers loaded on well-arranged asymmetrical unit cells to achieve the anisotropic or chiral EM features. Various configurations of the unit cell have been demonstrated can efficiently convert the polarization in transmission or reflection modes, such as split-ring resonator, arrow structure, I-shape and other hybrid structures. With the promotion of the research, the polarization converter obtains a great improvement on different directions. For example, the angular stability, i.e. the relationship of coefficient of polarization conversion (CPC) and incident angle, of the converter is increased to 45° []. The polarization conversion models contain not only linear cross-polarization [], but also linear polarization (LP) to circular polarization (CP) []. Broad- or multi- operation bandwidth have been also demonstrated []. In particular, with the help of microwave diodes, the artificial controllable polarization converter also has been explored.

In recently, multifunctional polarization converter attracted great interests, which can obtain both CPC and LP to CP conversion at different frequencies. Some works about reflective anisotropic metasurface has been reported for achieving multifunctional polarization converters. However, researches on multi-functional converters in transmissive mode are lacking.

In this paper, an anisotropic metasurface composed of a series of flower-liked unit cells separated by air spaces are designed with ability of CPC and LP-to-CP conversion for normal and oblique incidence. Linearly polarized incident wave is converted into cross-polarization transmission wave in the frequency band of 8.8 to 11.2 GHz with the lower limit of transmittance being 0.9. The angularly stable linear CPC is about 80° for incidece of low frequencies. Meanwhile, the CPC of LP-to-CP is dependent on the polarization of the incident. Although they show same polarization conversion at normal incident, the TM incident wave exhibits a better CPC as incident angle increased up to 60°.

- Unit cell design

Figure 1 depicted the schematic map of the unit cell, which is composed of flower-liked metallic strips loaded on the dielectric substrate with permittivity of 2.65 and loss tangent 0.003. The metallic strips are radially aligned, starts from r and ends at a. Specifically, the start points of the strips are linked together. The adjacent unit cells along y-axis are separated by air space with thickness of 2d. Positive z direction is defined as normal incidence. Symbol θ is the angle of wave vector with z-axis. Normal incidence means the EM wave propagating along θ = 0. The angle of the projection of wave vector on the xoy plane and the x-axis denoted as φ. When the incident EM waves are linearly polarized, coefficients of the polarization conversion are sensitive with φ due to the asymmetrical structure along z axis. The simulations carry on based on the method of finite time domain integration (FTDT). To facilitate understanding, Tji is defined as the transmission coefficient for ‘i’ polarized incident wave and ‘j’ polarized transmissive wave.

Figure.1 Schematic map of designed unit cell with parameters of P = 13, a = 6.3, h = 3, d =1.225 in unit of mm. The width of metallic arm sets to be 0.55 mm at initial.

Figures 2(a) show the coefficient of the co- and cross polarized transmissive wave, denoted as Txx , Tyx, Tyy, and Txy, under the x- and y-polarized normal incidences with condition of φ = 0. Obviously, none polarization conversions occur. For y-polarized waves, the metasurface is equal to a simple RC circuit with reflective resonance frequency around 11.5 GHz, which is mainly related with the feature of dielectric material and the air space. The situation is different for x-polarization incident. Under the influence of electric field, metallic arms of unit cell behavior like multipole dipoles coupled with each other. Two peaks originated from the multipole resonant appear around 8.3 GHz and 11.6 GHz under the given structural parameters. In general, the transmission coefficients are higher than 0.8 in the frequency range from 7.2 to 11.3 GHz. Furtherly, phases of the transmission waves are investigated, as shown in Fig.2(b). The red dash line Pyy and the blue solid line Pxx stand for phases of unconverted transmission waves, and the black dots line depicts the phase difference. It is found that the phase difference is about 90° around 7.7 GHz (marked with green circle) and about 180° around 10.27GHz (marked with purple circle), respectively. Mapping the areas to the Fig.2(a), we find that transmission coefficients of the two waves are approximate equal, suggesting the ability of multi-functional polarization conversion. To verify it, the electric field direction is set to along φ = 45°. Figure 2(c) shows the simulation results of the transmittance, where ‘u’ and ‘v’ are mapping to ‘x’ and ‘y’ directions after rotating 45°. It is obvious that the two polarized waves exhibit the same polarization conversion behavior, i.e., CPC. In particular, the coefficients of polarization conversion are greatly than 90% in the operation region of 8.8 ~11.1 GHz, corresponding to the area marked by purple circle. In other words, the metasurface is equal to a microwave half-plate in this case. Meanwhile, we find that the amplitudes of co- and cross polarized waves are equal at 7.7 GHz, applying an alternative polarization conversion manner. Thus, circular polarization states of the transmission wave are monitored. The simulation results are depicted in Fig.2(d). The two curves overlap well, except that they convert into different states. The u- and v-polarized waves converts to left-circular polarized (LCP) and right-circular polarized (RCP) wave, respectively. The peak appears at 8 GHz, falling into the area marked with green circle in Fig.2(b). The frequency region where the transmittance is higher than 0.9 is from 7.2 GHz to 8.8 GHz. Under this strategy, the metasurface can be regarded as a microwave quarter plate with capability of making a LP incident into a CP transmission wave. Via the relation between transmission complex coefficients of the linear t and the circular EM waves tcp, we can evaluate the behavior of circular polarization incident:

Where ‘+’ and ‘-’ stand for RCP and LCP waves, respectively. The calculation results are very similar to Fig.2(c), i.e., the CP incident converts to the cross polarized wave in range of 8.8 ~11.1 GHz. The results would not be depicted here, but in the following as part of angular stability of the converter.

Figure. 2 (a) and (b) are the transmittance amplitude and phase of EM incident waves with electric filed along x and y axis, respectively. Phase difference between the two waves appends in (b). (c) the transmittance of co- and cross polarization waves and (d) the transmittance of LP to CP conversion for u- and v- polarized incident waves.

- Angular stability

As an important feature of artificial polarization converter, the angular stability needs to be considered. The coefficients of polarization conversion for the designed metasurface are investigated as a function of frequency and incident angle θ under different situations. Figures 3(a) and (b) show the results under v- and u-polarized EM waves. It is found that the operation bandwidth holds about 2 GHz within ±30°, although slight red shift occurs. However, the bandwidth decreases abruptly to about 0.5 GHz around 8.6GHz as the incident angle increases to 85°. The coefficients for LP-CP conversion are displayed in Fig.3 (c) and (d). The performance of v-polarized to RCP polarized is better than that of u-polarization to RCP polarization. The operation bandwidth of 1GHz holds up to 55° and the coefficient is above 0.9 below 65° for v-polarized incident. While for u-polarized incident, the operation bandwidth of 1 GHz keeps at 45° and the coefficient reduces to 0.9 at 60°. If the coefficient threshold set to be 0.8, it is exciting that the converter can still work for the incident with frequency of 8.5GHz and angle above 85°. As a supplementary, conversion behaviors of the CP incident are studied. Only the coefficients of polarization conversion from LCP to RCP are displayed due to the reciprocity of EM waves. As shown in Fig. 3(e), The bandwidth at normal incident is about 2.3 GHz and the 2GHz operation bandwidth is at the region of incident angle between ±30°, same with cases under the LP incident. But the coefficient larger than 0.9 only can be guaranteed below 65°. The excellent angular stability is attributed to the multi-axis symmetrical of the unit cell on xoz plane. Whatever the incident angle is, we can always find metallic arms on the unit cell to couple with the electric field of the y-polarized EM incident. Figure 3(f) shows the transmittances of x- and y-polarized EM waves at different incident angles. It is obvious that the transmittance of x-polarized wave is reduced insensitively as incident angle larger than 50°, while a transmittance peak which is generated from the resonant of dipole, holds at low frequency for y-polarized waves at incident angle of 80°. The higher transmittance peak is suppressed due to the weakened coupling between dipoles at large incident angle. The high transmittance of y-polarized EM waves guarantees the angular stability of metasurface, despite the narrowed bandwidth.

Figure. 3 (a)-(e) are the coefficient of polarization conversion as a function of frequency and incident angle θ under different situations noted on top of the figures. (f) the transmittances of incident EM waves with x-polarization (dash-dot line) and y-polarization (real line) for the given incident angles of 0, 50°, and 80°.

- Influence of structural parameters

Generally, configurations of the metasurface are delicately designed to realize the desired functions. In other words, the performance is sensitive to the structural parameters. Here, influences of the structural parameters on the coefficient of polarization conversion are investigated in detail. As mentioned above, the metasurface can be treated as a RC circuit under the y-polarized incident waves. The capacitance is rather mainly dependent on the space between the adjacent layers, not the shape of the metallic unit cell for the given materials. However, for the x-polarization EM wave, due to the induced current by the electric field on the metallic arms, multiple dipoles appear and coupled to each other. Dipoles are formed by metallic arms which connects to shape of unit cell.

Figures 4 shows the coefficient of polarization conversion under different parameters. In Fig. 4(a), we display the relationship between coefficient of polarization conversion and f the number of the metallic arm n + 1. As n increases, the coupling of dipoles is enhanced, resulting in that the intrinsic characteristics of dipole is suppressed. Thus, the transmittance at low frequency are reduced and the operation bandwidth is narrowed. Meanwhile, the complex coupling appears at higher frequency around 14 GHz for n = 47. It is suggested that the bandwidth is a trade-off between dipole and coupling of dipoles. Thus, number of the metallic arms should be carefully adopted to achieve a broad operation band. It is also the main reason for Fig. 4(b), where the variable parameter is the width of the metallic arm expressed as 360×wd/(n+1). The width of metallic arms is increases with wd, resulting in the enhanced coupling of dipoles. In the case of wd = 0.04mm, the metallic arms are individual and not linked together. The resonance appears at a higher frequency that is out of the monitored range. When increasing a, the resonant frequency of dipole shifts to lower position and the coefficients increase slightly, as Fig.4(c) shows. On the contrary, as value of a decreases, the resonance of dipole moves to a higher frequency. It is easier to couple between dipoles, resulting a narrow bandwidth. Figure 4(d) depicts the influence of r. The operation bandwidth moves to a higher frequency and seems to be broadened as r reduces. However, the coefficients are suppressed a little. The reason is that although the length of dipole is increased, the increased part in fact is the proximal end of the arm which are linked together, leading to an enhancement of coupling between diploes not the diploes itself.

Figure. 4 Coefficients of polarization conversion under different structural parameters. （a）number of the metallic arm n (b) width of the metallic arm expressed as 360×wd/(n+1), where wd is defined the unit width per degree. (c) radius from center to the distal end of the arm a. (d) the radius from center to the proximal end of the arm r. Unit in mm.

- Conclusions